C/E3716 – UPPER SHOCK CROSSMEMBER

PARTS LIST:

1...40” Crossmember					4…Chassis tabs

4…5/8” to 1/2” Mis-alignment bushings		2…1/2” x 2-1/2” Bolts

2…Nylock nuts					2…1/2” Jam nuts

These instructions are just one way of properly installing Crossmember Kits.  Depending on your fabrication experience, you may find it easier or more convenient using other methods that accomplish the same results.  Every installation is slightly different, and we have attempted to structure these instructions to make your installation as easy as possible.

INSTALLATION:

1…Determine where you need to mount the crossmember.  All measurements should be made assuming that shocks are at ride height.  Alignment should be with the shocks straight up and down and side to side.  You may lean the shocks in/or forward if necessary.  Do not exceed 10 degrees.  Also, make sure you have enough spring clearance around the entire length of travel. Any amount of interference will cause damage to the shock, spring and mounts.

2…Measure the width of where the crossmember is to be placed.  Cut to length.

3…Place the crossmember in the predetermined location and tack weld into place.  Assemble the chassis tabs using the nuts and bolts supplied.  Make sure that you put the bolts in from the inside out.  Nuts should be on the frame side.  Use the mis-alignment bushings if you do not have 1-1/2” wide bearings in the shock end.

4…Tack weld the tabs into place on the crossmember.  Break the crossmember loose from the tack welds and pull from the car.  In a suitable location, final weld the tabs to the crossmember.

5…Re-install the crossmember in the proper location and double check the fitment of the shocks.  Once everything is in the proper location and clearances have been checked. Finally weld the crossmember into place.

6…Paint all surfaces for protection and

install shocks using the Nylock nuts.