Mid-Plate Mounting Instructions

These instructions are just one way of properly installing Engine/Transmission Mid-Plates.  Depending on your fabrication experience, you may find it easier or more convenient using other methods that accomplish the same results.  Every installation is slightly different, and we have attempted to structure these instructions to make your installation as easy as possible.

Applies to part numbers: 3696, 3696-2, 3699, 3704, 3706, 3707, 3709

1...Start the installation by making a cardboard template of the mid-plate (you can use the box it is packed in).  Next fasten the template to rear of the motor.

2...Place motor in the chassis in the desired location.  Make sure that the motor is in proper alignment with the rear end.

3...Carefully mark and trim the cardboard template to fit the chassis.

4...Using the cardboard template, mark and trim the mid-plate accordingly.

5…Mount the newly trimmed mid-plate on to the motor and re-check the fit in the chassis.

6…Clamp the mounting brackets to the rear of the plate and tack weld to the chassis. Chassis Engineering offers two kits for mid-plate mounting, the C/E3689 Pro Style kit and C/E3389 Tubular Mid-Mount Loop.

7…Pull the motor back out of the chassis and finish welding the brackets into place.

8…While the motor is out of the chassis, mount the starter with the mid-plate in place to be sure that everything fits correctly.  Grind plate as necessary to create the correct fit and clearances.

Note: Front motor plate and engine travel limiters are recommended for correct engine mounting and stability.

0925